Data set Description

The dataset directory contains two primary files:

- Raw dataset – the unprocessed measurements collected directly from the milling experiments.

- Training dataset – a refined and preprocessed version of the raw data prepared specifically for model training.

- Experimental Data Collection

The raw dataset was generated from 25 extensive surface-milling experiments conducted on S45C carbon steel. During these experiments, two key cutting parameters were systematically varied to build a representative dataset:

- Spindle speed: adjusted from 1920 to 2450 rpm

- Depth of cut: varied between 1 and 5 mm

By altering these operating conditions across multiple trials, a diverse set of spindle power responses was captured to support accurate model training and evaluation.

- Machining Setup

All milling operations were performed on an NMV76A three-axis vertical milling machine, manufactured by YEONG CHIN MACHINERY INDUSTRIES CO., LTD. This industrial-grade machine ensured stable and repeatable cutting conditions throughout the experiments.

- Sensor and Measurement Details

Spindle power and electrical parameters were measured using a CPM-12D Multifunctional Power Meter (DIN-rail type). The CPM-12D is a high-accuracy metering device capable of monitoring both single-phase and three-phase systems. It measures a comprehensive set of electrical parameters, including:

- Voltage (V)

- Current (I)

- Active power (P)

- Reactive power (Q)

- Energy consumption (kWh, kVarh)

- Additional derived electrical quantities

This precise and reliable measurement setup ensured that the collected spindle power data accurately reflected the machine’s behavior under different cutting conditions.